## Application Note - AN005

## Perl Control of Quarch Modules

## Aug, 2015

Contents

Requirements	3

Set-up	3

Installing Required Libraries	4

Installing ActiveState Perl	4

Installing Win32 Serial Port module for Perl (Windows Only)	4

Installing Device Serial Port module for Perl (Linux Only)	5

Installing Net Telnet module for Perl (Linux Only)	6

Connection discovery and set-up	7

Test Description	9

## Perl Control

The aim is to establish a connection with the module and send test commands using the “Perl Control Examples.pl” script. The main objective is to

- Show how to connect to a module

- Run example commands depending on the type of the module (hot plug module, switch module or power margining module).

# Requirements

- Quarch Module (Hot-plug Module, Switch Module and Power Margining Module)

- Host PC

- Power supply  for the relevant Module

- Perl and Serial Port Extension

- LAN/Serial Connection to the module

## Set-up

Host PC

Quarch Module

Serial or LAN

# Installing Required Libraries

You may have some or all of these items installed.  In this case you can ignore those steps.

## Installing ActiveState Perl

Head to http://www.activestate.com/activeperl/downloads and download “ActivePerl 5.20 for windows (x86)” and install.

## Installing Win32 Serial Port module for Perl (Windows Only)

Head to http://search.cpan.org/~bbirth/Win32-SerialPort-0.22/lib/Win32/SerialPort.pm and download “Win32-SerialPort-0.22.tar.gz”.

Extract the archive into a subdirectory located in the root directory of your drive “C:\”.

Through command prompt, navigate to the directory where the archive was extracted.

Execute “perl makefile.pl” and wait for completion. Then, execute “perl nomake_test” and wait for completion. Finally, run “perl nomake_install”.

## Installing Device Serial Port module for Perl (Linux Only)

Head to http://search.cpan.org/~cook/Device-SerialPort-1.002/SerialPort.pm and download “Device-SerialPort-1.002.tar.gz”

Extract the archive into a folder in the home directory.

Through terminal navigate to the extracted folder and execute “perl Makefile.PL”.

Once completed, execute “make”, followed by “make test”, followed by “make install”.

## Installing Net Telnet module for Perl (Linux Only)

Head to http://search.cpan.org/~jrogers/Net-Telnet-3.04/lib/Net/Telnet.pm and download “Net-Telnet-3.04.tar.gz”

Extract the archive into a folder in the home directory.

Through terminal navigate to the extracted folder and execute “perl Makefile.PL”.

Once completed, execute “make”, followed by “make test”, followed by “make install”.

# Connection discovery and set-up

The IP of your device can be found by using the "CONFig:ETHernet:IP?" command over Serial or USB. Alternatively, if you have netBIOS names on your network, you can also use the serial number of the module for connection "Telnet:QTLxxxx-xx-xxx".

This script will run using either a Serial or a Telnet connection. This can be set up accordingly by editing the script and commenting out the appropriate line of code.

- For example, to run the script over Ethernet, modify the following line and enter the correct IP address of the device;

$Connection = OpenConnection("Telnet:192.168.1.210")

and comment out,

$Connection = OpenConnection("Serial:COM4")

- For use over serial, edit the following line to match the COM port associated with the device

$Connection = OpenConnection("Serial:COM4")

and comment out

$Connection = OpenConnection("Telnet:192.168.1.210")

Note: Under linux environments, Serial ports must be changed to the appropriate format, e.g.

$Connection = OpenConnection("Serial:/dev/ttyS3");

The Perl script will send test commands to the Quarch Module. The script will display the module connected and send commands to the module.

Depending on the module connected, you must comment in the correct test example.

- QuarchHotPlugExample – Works with all standard hot-plug modules

- QuarchSwitchExample – Works with SAS switch modules

- QuarchPowerMarginingExample – Works with Programmable Power Modules

These are represented by these lines of code executing the functions mentioned above

QuarchHotPlugExample(Connection)

QuarchSwitchExample(Connection)

QuarchPowerMarginingExample(Connection)

Again it is important to note that python is sensitive to indentation so ensure the indenting is kept consistent with the rest of the code.

[Note: Please refer to the technical manuals for the list of the commands applicable to the module attached.]

## Test Description

Each test function will:

- Demonstrate the sending of commands to the module

- Show how to read back and use a response to a command

Each example is different (depending on the functions available on the module) and tries to show a genuine, practical test that you can perform.