Quarch Technology Ltd

AN-024

Application Note

QPS Window Averaging Example

For use with:

HD PPM and PAM

QTL1999 / QTL1995 / QTL2312

# Change History

| 1.0 | October 2021 | Initial Release |
| --- | --- | --- |

Contents

Change History	2

Introduction	4

WARNING	Error! Bookmark not defined.

Requirements	Error! Bookmark not defined.

Installation	Error! Bookmark not defined.

Equipment Setup	Error! Bookmark not defined.

Calibration Process	Error! Bookmark not defined.

Power Module Selection	Error! Bookmark not defined.

Select Action	Error! Bookmark not defined.

Select Calibration Instrument	Error! Bookmark not defined.

Calibration Sections	Error! Bookmark not defined.

Switch Requests	Error! Bookmark not defined.

Calibration Output	Error! Bookmark not defined.

# Introduction

- Post processing of captured data can be very useful to get the exact results you need for comparison.

- This simple example implements a sliding average window over a data set, looking for a worst-case power measurement.  The input data is in CSV form, exported from QPS (Though it will work with any similar data set).

- The output is a single value for the worst case average found in any window.  This is a common way to find the peak power consumed by a device, while excluding brief noise spikes.

- The window size can be set to achieve the result you need.  A short window of 100uS could be used to reject noise and brief spikes, while a window of 100mS to 1 second might be useful for considering the overall view of a workload.

- The process time depends on the CSV file size.  A 26GB CSV file in our test run took 10 minutes to process.

- Note that this example does NOT require Quarch hardware to run, only the data captured from it previously.

# Data for processing

- First you need data to process.  This can be dumped to file via automated capture using QIS or QPS, examples are here:

- https://quarch.com/file/an-012-qis-python-control-of-power-modules/

- https://quarch.com/file/an-015-automating-qps/

- It can also be taken manually from QPS by exporting to CSV format, which is found in the File->Export menu.

# Running the script

Run the supplied WindowAverageExample.py using python

> python WindowAverageExample.py

- Ensure the test_data.csv file supplied is still present

# Supply your own data

Simple switch out the path to the test data to supply your own

- To run, you must ensure:

Your window length is supplied in the same units as the CSV time column

The column name to process is correct

The CSV delimiter is correct for the CSV file format (normally a comma)