Quarch Technology Ltd

AN-024

Application Note

HD PPM Pattern Example

For use with:

HD Power Modules

# Change History

| 1.0 |  | Initial Release |
| --- | --- | --- |
| 1.1 | November 2022 | Fixed python warning |

Contents

Change History	2

Introduction	4

Modules Supported	5

System Supported	5

Application Note Example Files	5

Installation and setup	5

Python install	5

https://docs.python.org/3/using/windows.html#excursus-setting-environment-variables	5

QuarchPy library install	6

Power module setup	8

Running the example	9

# Introduction

The pattern generator in our PPM (Programmable Power Modules) is a very useful feature which can create a huge range of power test cases:

Ramps

Glitches

Power loss

Brownout

Ripple

- Here we show how to create a pattern using a script.  In this case we will create a ‘chirp’ on the rail.  This can be increased in magnitude until a drive failure is detected.

# Modules Supported

HD power modules

QTL1995

QTL1999

XLC power modules (Supports this feature, but not specifically tested with this note)

QTL1824

QTL1847

# System Supported

This example is written and tested on Windows, though could be used on Linux and MacOS. It currently requires Python 3.x

## Application Note Example Files

The AN-024.zip should be extracted to your preferred location.

| PatternExample.py | Main python file to execute |
| --- | --- |

# Installation and setup

## Python install

If you do not already have Python 3.x installed, download and install it from:

https://www.python.org/downloads/

Under Windows it is helpful to make sure the Python installation directory and PythonXX\Scripts are included in the PATH environment variable. See:

https://docs.python.org/3/using/windows.html#excursus-setting-environment-variables

## QuarchPy library install

The Quarch Python package can be installed from the Python web repository (assuming you have internet access) or via the download from our website.

Quarchpy will also install a version of Quarch Power Studio

### Web Install

From the command line:

>pip install quarchpy

If this fails, your path to “pip” may not be set, you can instead run:

>python –m pip install quarchpy

### Local Install

If you want to install from a downloaded folder, ensure the folder is unzipped to a local disk, navigate to the folder containing the setup.py file and run (noting the ‘.’ on the end):

>pip install quarchpy .

If this fails, your path to ‘pip’ may not be set, you can instead run:

>python –m pip install quarchpy .

### Upgrade

If you already have QuarchPy installed, you will get a failure message. If you want to upgrade to a new version, you need to add the ‘--upgrade’ command:

>pip install --upgrade quarchpy

The --upgrade command can similarly be used in any of the other examples, to load from a local install folder.

## Power module setup

Connect the power module to the test PC and the output of the power module to the drive under test.

Customer Drive

USB/LAN Connection

Power Module

Customer Test PC

# Running the example

Execute the script and the pattern will be sent to the device and run.  A simple loop modifies the pattern each time, to increase the magnitude of the ripple.

Here is an example of the pattern when opened in Quarch Power Studio.

The example script just creates the pattern and runs it, but you could easily add in your own code to verify that the drive handled each run of the pattern as expected.